F10 SDK

Quick Start Guide

Version: 1.0       Date:  Dec. 2020

Telepower Communication Co., Ltd.

© Telepower Communication Co., Ltd.

CONTENT

NFC	2

Relay	2

Method：	2

Rs485	3

Switch to receive mode：	3

Switch to send mode：	3

LED	3

Open Method：	3

Close Method：	4

Wiegand	4

method：	4

Receive Wiegand Data：	5

Lan	5

Method：	5

NFC

Enter the NFC test interface, close the card to the card recognition area, and if the card information is displayed, the NFC test is successful; If the message "Device does not support NFC" is displayed, there is a problem with the device's NFC function; If the "please go to set inside the NFC switch" is displayed, the device supports NFC, but it is not enabled, please go to the settings first to enable.

Use android common NFC API.

Relay

- Method：

PosUtil.setRelayPower(int number);

Parameters:

number: 1 – open , 0 – close

Call Posutil's static method Setrelaypower () method, set the parameter value to 1, if the return value is 0 to open the relay successfully, if not 0 then open failed.

Rs485

- Switch to receive mode：

PosUtil.setRs485Status(0);

Call Posutil static method Setrs485status (), set the parameter value to 0, if the return value is 0, successfully switch to receive mode, if not 0 the switch fails.

- Switch to send mode：

PosUtil.setRs485Status(1);

Call Posutil static method Setrs485status (), set the parameter value to 1, if the return value is 0 successfully switch to send mode, if not 0 the switch fails.

If you want to send or receive data,please use android common Serial port API.

LED

- Open Method：

PosUtil.controlledBright(int type, int progress)

Parameters:

type: 0 – red led   1 – green led  2 – blue led  3 – white led

progress: brightness ,range is 0-255

Call led static method controlledBright(),by introducing different types, you can turn on different lights and adjust the brightness. You can turn on four lights at the same time. Different combinations can produce new colors.

- Close Method：

PosUtil.controlledBright (4,1)

Parameters:

Fixed value

Call led static method controlledBright(),if the return value is 0, means open led success, if not 0 means open led failed.

Wiegand

- method：

PosUtil.getWg26Status(long cardid);

PosUtil.getWg34Status(long cardid);

Parameters:

cardid: Decimal card number

PosUtil.sendSpecialWG(String cardNumBin)

Parameters:

Send custom Wigan signal(up to 64 bits)

cardNumBin: Binary string to send signal

Call Wiegand static method ,if the return value is 0 successfully means send success, if not 0 means send failed.

- Receive Wiegand Data：

PosUtil posutil = new PosUtil(context);

posutil.registerBroadcastWiegandInput();//Register broadcast

posutil.getWiegandInput(new WiegandInputListener() {// Listen for Wigan input

@Override

public void wiegandInput(byte[] inputData) {

// TODO Auto-generated method stub

// Data entered by Wigan: inputData

String data = new String(inputData);

}

});

posutil.unRegisterBroadcastWiegandInput();//At the end of the call, the broadcast is unregistered

Lan

- Method：

PosUtil.setLanPower(int number);

Parameters:

number: 1 – power on , 0 – power off

Call Posutil's static method SetLanpower () method, set the parameter value to 1, if the return value is 0 to power on lan successfully, if not 0 then power on failed.

Telepower Communication Co., Ltd

For more information，pls visit our website：www.telpo.com.cn

Tel： +(00)86-757-86227898

Email：business@telpo.com

Add：Foshan City, Guangdong Province, China

© Copyright 2020 Telepower Communication Co., Ltd. All Rights Reserved.